SRM VALLIAMMAI ENGINEERING OLLEGE

(An Autonomous Institution)

SRM Nagar, Kattankulathur – 603 203

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

QUESTION BANK

V SEMESTER

PEC101 WIDE BANDGAP DEVICES

Regulation – 2023

Academic Year 2026-2027 (Odd)

Prepared by

Dr. N. Jothy, Asst. Professor (Sr. Grade)/ECE

SRM VALLIAMMAI ENGINEERING COLLEGE

(An Autonomous Institution)

SRM Nagar, Kattankulathur – 603 203.

DEPARTMENT OF ELECTRONICS & COMMUNICATION ENGINEERING

QUESTION BANK

SUBJECT	: PEC101 WIDE BANDGAP DEVICES

SEM / YEAR:  V/ III-year B.E.

| UNIT I WBG DEVICES AND THEIR APPLICATION IN REAL WORLD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Review of semiconductor basics - Operation and characteristics of the SiC Schottky Barrier Diode- SiC DMOSFET and GaN HEMT- Review of Wide bandgap semiconductor technology -Advantages and disadvantages. |  |  |  |  |  |
| PART – A |  |  |  |  |  |
| Q.No | Questions |  | CO | BT Level | Competence |
| 1. | Define a semiconductor. |  | CO1 | BTL 1 | Remembering |
| 2. | What is the bandgap energy of silicon carbide (SiC)? |  | CO1 | BTL 1 | Remembering |
| 3. | Explain why SiC is suitable for high-temperature applications. |  | CO1 | BTL 2 | Understanding |
| 4. | List two wide bandgap semiconductor materials. |  | CO1 | BTL 1 | Remembering |
| 5. | Compare the switching characteristics of SiC SBD and PN diode. |  | CO1 | BTL 2 | Understanding |
| 6. | Why does GaN HEMT offer high electron mobility? |  | CO1 | BTL 2 | Understanding |
| 7. | What is the full form of DMOSFET? |  | CO1 | BTL 1 | Remembering |
| 8. | State one application of GaN HEMT. |  | CO1 | BTL 1 | Remembering |
| 9. | Describe the working principle of a SiC Schottky diode. |  | CO1 | BTL 2 | Understanding |
| 10. | Differentiate between intrinsic and extrinsic semiconductors. |  | CO1 | BTL 2 | Understanding |
| 11. | Write any two advantages of SiC Schottky Barrier Diode. |  | CO1 | BTL 1 | Remembering |
| 12. | What is the typical operating temperature range of SiC devices? |  | CO1 | BTL 1 | Remembering |
| 13. | Mention any two characteristics of wide bandgap semiconductors. |  | CO1 | BTL 1 | Remembering |
| 14. | Name two differences between Si MOSFET and SiC MOSFET. |  | CO1 | BTL 1 | Remembering |
| 15. | What is a Schottky barrier? |  | CO1 | BTL 1 | Remembering |
| 16. | What type of carriers conduct current in a Schottky diode? |  | CO1 | BTL 1 | Remembering |
| 17. | What is the primary advantage of using GaN over Si in power electronics? |  | CO1 | BTL 1 | Remembering |
| 18. | Why are wide bandgap materials preferred in high-frequency applications? |  | CO1 | BTL 2 | Understanding |
| 19. | Explain the term “2D Electron Gas” in GaN HEMT. |  | CO1 | BTL 2 | Understanding |
| 20. | Illustrate how a SiC DMOSFET operates in the on-state. |  | CO1 | BTL 2 | Understanding |
| 21. | What type of carriers conduct current in a Schottky diode? |  | CO1 | BTL 1 | Remembering |
| 22. | Compare the conduction mechanism of Schottky diode and PN diode. |  | CO1 | BTL 2 | Understanding |
| 23. | Why is gate drive design more critical in GaN devices compared to Si? |  | CO1 | BTL 2 | Understanding |
| 24. | Explain the reason for low switching losses in GaN HEMT. |  | CO1 | BTL 2 | Understanding |
| PART- B |  |  |  |  |  |
| 1. | (i) Explain the intrinsic and extrinsic semiconductors with examples. (ii) Describe the working of a PN junction with the help of energy band diagrams. | (7) (9) | CO1 | BTL 3 | Applying |
| 2. | (i) Discuss the characteristics of n-type and p-type semiconductors. (ii) Compare direct and indirect bandgap semiconductors. | (8) (8) | CO1 | BTL 4 | Analyzing |
| 3. | (i) Define mobility and conductivity. Derive the relation between them. (ii) Explain the effect of temperature on carrier concentration in semiconductors. | (10) (6) | CO1 | BTL 4 | Analyzing |
| 4. | (i) Explain the construction and working principle of SiC Schottky Barrier Diode. (ii) Discuss the I-V characteristics and switching behavior of SiC SBD. | (8) (8) | CO1 | BTL 3 | Applying |
| 5. | (i) Compare the performance of Si-based SBD and SiC SBD in high-frequency applications. (ii) List and explain the advantages of SiC SBD over conventional diodes. | (8) (8) | CO1 | BTL 4 | Analyzing |
| 6. | (i) Draw and explain the energy band diagram of a Schottky junction. (ii) Describe the role of barrier height and its effect on diode performance. | (9) (7) | CO1 | BTL 4 | Analyzing |
| 7. | (i) Describe the structure and working of a SiC DMOSFET. (ii) Compare Si MOSFET and SiC DMOSFET in terms of on-resistance and thermal performance. | (10) (6) | CO1 | BTL 3 | Applying |
| 8. | (i) Explain the transfer and output characteristics of SiC DMOSFET. (ii) Discuss the benefits of using SiC DMOSFETs in electric vehicles and power electronics. | (8) (8) | CO1 | BTL 3 | Applying |
| 9. | (i) Describe the breakdown mechanism in SiC MOSFETs. (ii) Highlight the challenges in gate oxide reliability in SiC DMOSFETs. | (6) (10) | CO1 | BTL 3 | Applying |
| 10. | (i) Explain the construction and operation of a GaN HEMT. (ii) Discuss the significance of 2DEG in GaN HEMT and how it enhances device performance. | (8) (8) | CO1 | BTL 3 | Applying |
| 11. | (i) Compare GaN HEMT and Si MOSFET in terms of switching speed and gate drive requirements. (ii) Illustrate the output and transfer characteristics of GaN HEMT. | (8) (8) | CO1 | BTL 3 | Applying |
| 12. | (i) Analyze the thermal and high-frequency behavior of GaN HEMT. (ii) Explain the design challenges in enhancement-mode GaN HEMTs. | (8) (8) | CO1 | BTL 4 | Analyzing |
| 13. | (i) Define wide bandgap semiconductors. Give examples and explain their properties. (ii) Compare Si, SiC, and GaN in terms of bandgap, mobility, and thermal conductivity. | (9) (7) | CO1 | BTL 3 | Applying |
| 14. | (i) Discuss the role of wide bandgap semiconductors in high-power and high-temperature electronics. (ii) Explain how WBG semiconductors enable smaller and lighter systems. | (8) (8) | CO1 | BTL 4 | Analyzing |
| 15. | (i) Analyze the key differences in fabrication between Si and WBG devices. (ii) Explain how substrate quality affects the performance of SiC and GaN devices. | (9) (7) | CO1 | BTL 4 | Analyzing |
| 16. | (i) List and explain the advantages of using SiC and GaN devices over silicon. (ii) Highlight the disadvantages and limitations of wide bandgap devices in commercial deployment. | (8) (8) | CO1 | BTL 3 | Applying |
| 17. | (i) Discuss the cost and reliability issues in SiC and GaN fabrication. (ii) Analyze the role of packaging and thermal management in WBG devices. | (8) (8) | CO1 | BTL 4 | Analyzing |

| UNIT II SWITCHING CHARACTERIZATION OF WBG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Turn-on and Turn-off characteristics of the device- Hard switching loss analysis- Double pulse test set-up. |  |  |  |  |  |
| PART – A |  |  |  |  |  |
| Q.No | Questions |  | CO | BT Level | Competence |
| 1. | Define turn-on time of a switching device. |  | CO2 | BTL 1 | Remembering |
| 2. | What is meant by turn-off delay time? |  | CO2 | BTL 1 | Remembering |
| 3. | Explain why switching losses occur during turn-on and turn-off transitions. |  | CO2 | BTL 2 | Understanding |
| 4. | Describe how the overlap of voltage and current during switching causes power loss. |  | CO2 | BTL 2 | Understanding |
| 5. | State the main components of total switching time. |  | CO2 | BTL 1 | Remembering |
| 6. | List any two parameters that affect switching losses. |  | CO2 | BTL 1 | Remembering |
| 7. | Why is turn-on loss generally higher than turn-off loss in some devices? |  | CO2 | BTL 2 | Understanding |
| 8. | Compare turn-on and turn-off characteristics of a MOSFET. |  | CO2 | BTL 2 | Understanding |
| 9. | What is hard switching in power electronic devices? |  | CO2 | BTL 1 | Remembering |
| 10. | Name the four intervals of a switching transition. |  | CO2 | BTL 1 | Remembering |
| 11. | Explain how hard switching increases device stress. |  | CO2 | BTL 2 | Understanding |
| 12. | Discuss the importance of gate resistance in shaping switching behavior. |  | CO2 | BTL 2 | Understanding |
| 13. | Define switching loss in a power semiconductor. |  | CO2 | BTL 2 | Understanding |
| 14. | What is the purpose of the double pulse test? |  | CO2 | BTL 2 | Understanding |
| 15. | Why is the double pulse test preferred over continuous switching tests for loss analysis? |  | CO2 | BTL 1 | Remembering |
| 16. | How does inductive load affect switching transitions in a test setup? |  | CO2 | BTL 2 | Understanding |
| 17. | Which device parameters are extracted using the double pulse test? |  | CO2 | BTL 1 | Remembering |
| 18. | List two advantages of using the double pulse test. |  | CO2 | BTL 1 | Remembering |
| 19. | Explain how dv/dt and di/dt affect device reliability during switching. |  | CO2 | BTL 1 | Remembering |
| 20. | Describe the role of a clamping circuit in a double pulse test. |  | CO2 | BTL 1 | Remembering |
| 21. | What type of waveform is typically captured in a double pulse test? |  | CO2 | BTL 2 | Understanding |
| 22. | Name any two instruments required for conducting a double pulse test. |  | CO2 | BTL 2 | Understanding |
| 23. | Why is accurate timing critical in a double pulse test setup? |  | CO2 | BTL 1 | Remembering |
| 24. | Explain how switching energy loss is calculated from double pulse test waveforms. |  | CO2 | BTL 1 | Remembering |
| PART- B |  |  |  |  |  |
| 1. | (i) Explain the switching behavior of a power MOSFET during turn-on and turn-off. (ii) Sketch and describe the typical voltage and current waveforms observed during turn-on and turn-off transitions. | (8) (8) | CO2 | BTL 4 | Analyzing |
| 2. | (i) Define and explain the terms: delay time, rise time, fall time, and storage time. (ii) Describe how gate resistance affects switching times of a power device. | (8) (8) | CO2 | BTL 3 | Applying |
| 3. | (i) Explain the impact of junction temperature and device capacitance on switching behavior. (ii) Analyze the effect of parasitic inductance on voltage overshoot during switching. | (8) (8) | CO2 | BTL 4 | Analyzing |
| 4. | (i) Describe how switching losses occur in a power semiconductor during hard switching. (ii) Derive the expression for total switching loss in a single switching cycle. | (8) (8) | CO2 | BTL 4 | Analyzing |
| 5. | (i) Explain why turn-on loss is typically higher than turn-off loss in some devices. (ii) Illustrate the switching loss distribution using waveforms. | (7) (9) | CO2 | BTL 3 | Applying |
| 6. | (i) Define hard switching and soft switching with examples. (ii) Explain the drawbacks of hard switching in power converters. | (8) (8) | CO2 | BTL 3 | Applying |
| 7. | (i) Describe the significance of dv/dt and di/dt during switching transitions. (ii) Explain how these parameters affect device stress and EMI. | (8) (8) | CO2 | BTL 4 | Analyzing |
| 8. | (i) Explain the purpose and working of a snubber circuit during switching. (ii) Discuss how snubber design affects switching losses. | (8) (8) | CO2 | BTL 3 | Applying |
| 9. | (i) Describe the working principle of a double pulse test setup. (ii) List and explain the key components used in a double pulse test circuit. | (10) (6) | CO2 | BTL 4 | Analyzing |
| 10. | (i) Describe the switching characteristics of an IGBT. (ii) Compare the switching performance of IGBT with a power MOSFET. | (8) (8) | CO2 | BTL 3 | Applying |
| 11. | (i) Sketch and explain the voltage and current waveforms captured during a double pulse test. (ii) Describe how switching energy is calculated using these waveforms. | (8) (8) | CO2 | BTL 3 | Applying |
| 12. | (i) Explain the need for using a double pulse test to evaluate switching losses. (ii) Describe how the test helps in comparing different devices like SiC, GaN, and Si. | (8) (8) | CO2 | BTL 4 | Analyzing |
| 13. | (i) Describe the test procedure for a double pulse test on a SiC MOSFET. (ii) Explain the measurement challenges involved in capturing fast switching waveforms. | (8) (8) | CO2 | BTL 4 | Analyzing |
| 14. | (i) Discuss how varying gate resistance affects switching loss and speed. (ii) Show the changes in waveform due to increased gate resistance using sketches. | (6) (10) | CO2 | BTL 4 | Analyzing |
| 15. | (i) Compare the switching losses of Si, SiC, and GaN devices using theoretical data. (ii) Explain how wide bandgap devices reduce hard switching losses. | (8) (8) | CO2 | BTL 4 | Analyzing |
| 16. | (i) Explain how switching losses lead to thermal stress in power devices. (ii) Describe the importance of thermal management in high-frequency hard switching circuits. | (7) (9) | CO2 | BTL 3 | Applying |
| 17. | (i) Discuss how switching characteristics influence the design of high-frequency converters. (ii) Explain methods to minimize switching losses through design optimization. | (9) (7) | CO2 | BTL 3 | Applying |

| UNIT III DRIVERS FOR WIDE BAND GAP DEVICES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Gate driver- Impact of gate resistance- Gate drivers for wide bandgap power devices - Transient immunity integrated gate drivers |  |  |  |  |  |
| PART - A |  |  |  |  |  |
| Q.No | Questions |  | CO | BT Level | Competence |
| 1. | What is a gate driver in power electronics? |  | CO3 | BTL 1 | Remembering |
| 2. | List two functions of a gate driver circuit. |  | CO3 | BTL 1 | Remembering |
| 3. | Define gate resistance. |  | CO3 | BTL 1 | Remembering |
| 4. | What is the typical range of gate voltage for SiC MOSFETs? |  | CO3 | BTL 1 | Remembering |
| 5. | Explain how gate resistance affects switching speed. |  | CO3 | BTL 2 | Understanding |
| 6. | Why do GaN and SiC devices require fast gate drivers? |  | CO3 | BTL 2 | Understanding |
| 7. | Name any two types of gate drivers. |  | CO3 | BTL 1 | Remembering |
| 8. | List two key characteristics of a good gate driver. |  | CO3 | BTL 1 | Remembering |
| 9. | Describe the effect of increasing gate resistance on EMI and switching losses. |  | CO3 | BTL 2 | Understanding |
| 10. | Differentiate between gate drivers for Si and WBG devices. |  | CO3 | BTL 2 | Understanding |
| 11. | What is transient immunity in gate drivers? |  | CO3 | BTL 1 | Remembering |
| 12. | Give two reasons why gate drivers are needed for wide bandgap devices. |  | CO3 | BTL 1 | Remembering |
| 13. | Explain the purpose of Miller clamp in gate driver circuits. |  | CO3 | BTL 2 | Understanding |
| 14. | Name two WBG devices that require specialized gate drivers. |  | CO3 | BTL 1 | Remembering |
| 15. | Why is precise timing important in gate drivers for fast-switching devices? |  | CO3 | BTL 2 | Understanding |
| 16. | What is the role of bootstrap circuitry in a gate driver? |  | CO3 | BTL 1 | Remembering |
| 17. | State one advantage of isolated gate drivers. |  | CO3 | BTL 1 | Remembering |
| 18. | Explain the role of gate drive strength (source/sink current) in switching control. |  | CO3 | BTL 2 | Understanding |
| 19. | State one advantage of isolated gate drivers. |  | CO3 | BTL 1 | Remembering |
| 20. | List any two parameters affected by gate resistance. |  | CO3 | BTL 1 | Remembering |
| 21. | Explain the role of gate drive strength (source/sink current) in switching control. |  | CO3 | BTL 2 | Understanding |
| 22. | Why is precise timing important in gate drivers for fast-switching devices? |  | CO3 | BTL 2 | Understanding |
| 23. | How does a transient event affect gate driver performance? |  | CO3 | BTL 2 | Understanding |
| 24. | Describe how integrated gate drivers improve transient immunity. |  | CO3 | BTL 2 | Understanding |
| PART- B |  |  |  |  |  |
| 1. | (i) Explain the key functions and requirements of a gate driver circuit for power electronic switches. (ii) Compare high-side vs low-side gate driving techniques with suitable circuit examples. | (8) (8) | CO3 | BTL 3 | Applying |
| 2. | (i) Describe the effect of gate resistance on switching speed and electromagnetic interference (EMI) in power converters. (ii) How does gate resistance influence device protection and thermal performance? Illustrate with waveforms. | (9) (7) | CO3 | BTL 4 | Analyzing |
| 3. | (i) What are the critical design parameters for selecting a gate driver IC for high-power applications? (ii) How do propagation delay and drive strength of a gate driver affect converter performance. | (8) (8) | CO3 | BTL 4 | Analyzing |
| 4. | (i) Compare gate driving requirements of silicon MOSFETs with those of SiC and GaN devices. (ii) Discuss the challenges in designing gate drivers for wide bandgap devices. | (7) (9) | CO3 | BTL 3 | Applying |
| 5. | (i) Explain the impact of high dV/dt and dI/dt of WBG devices on gate driver design. (ii) What techniques are used to mitigate these effects? Explain with practical examples. | (8) (8) | CO3 | BTL 3 | Applying |
| 6. | (i) Describe common gate driver protection features such as under-voltage lockout (UVLO) and desaturation detection. (ii) Explain active Miller clamping and its role in preventing false turn-on. | (8) (8) | CO3 | BTL 3 | Applying |
| 7. | (i) Define common-mode transient immunity (CMTI) and its relevance in gate driver performance. (ii) How can PCB layout and isolation improve transient immunity in integrated gate drivers? | (8) (8) | CO3 | BTL 3 | Applying |
| 8. | (i) Compare different isolation methods used in gate drivers: magnetic, capacitive, and optical. (ii) Explain how isolated gate drivers enhance safety and noise immunity in power converters. | (10) (6) | CO3 | BTL 3 | Applying |
| 9. | (i) Discuss the unique gate drive requirements of GaN HEMTs. (ii) Compare enhancement-mode GaN vs. cascode GaN gate drive strategies. | (10) (6) | CO3 | BTL 3 | Applying |
| 10. | (i) Describe turn-on and turn-off gate voltages required for SiC MOSFETs. (ii) Explain the role of negative gate bias and its impact on performance and reliability. | (8) (8) | CO3 | BTL 4 | Analyzing |
| 11. | (i) Explain the working and advantages of dual-channel gate drivers in half-bridge topologies. (ii) Discuss shoot-through prevention techniques in gate driver circuits. | (9) (7) | CO3 | BTL 3 | Applying |
| 12. | (i) List and explain PCB layout considerations for high-speed gate driver circuits. (ii) Discuss the role of Kelvin source connections in improving gate driver performance. | (8) (8) | CO3 | BTL 4 | Analyzing |
| 13. | (i) Explain the concept and advantages of integrated gate drivers in modern power electronics. (ii) Describe how smart gate drivers adapt to device conditions and improve system efficiency. | (8) (8) | CO3 | BTL 3 | Applying |
| 14. | (i) Discuss the role of digital control and programmability in gate driver design. (ii) Explain how programmable gate resistance and timing can optimize WBG device performance. | (10) (6) | CO3 | BTL 4 | Analyzing |
| 15. | (i) Explain how gate driver power dissipation is estimated and managed. (ii) Describe thermal management techniques for gate driver ICs in high-frequency applications. | (8) (8) | CO3 | BTL 4 | Analyzing |
| 16. | (i) List key parameters to consider when selecting a gate driver for a SiC-based inverter. (ii) Evaluate a commercial gate driver (like TI UCC21710 or Infineon 1EDC) and explain its features. | (7) (9) | CO3 | BTL 4 | Analyzing |
| 17. | (i) Present a case study on gate driver implementation in a high-voltage DC-DC converter using SiC MOSFETs. (ii) Identify challenges and solutions related to transient immunity and EMI in that application. | (8) (8) | CO3 | BTL 4 | Analyzing |

| UNIT IV HIGH FREQUENCY DESIGN COMPLEXITY AND PCB DESIGNING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Effects of parasitic inductance - Effects of parasitic capacitance - EMI filter design for high frequency power converters -High frequency PCB design - Conventional power loop design - High frequency power loop optimization - Separation of power from signal PCB. |  |  |  |  |  |
| PART – A |  |  |  |  |  |
| Q.No | Questions |  | CO | BT Level | Competence |
| 1. | Define parasitic inductance in a power converter circuit. |  | CO4 | BTL 1 | Remembering |
| 2. | State two sources of parasitic inductance in a PCB layout. |  | CO4 | BTL 1 | Remembering |
| 3. | Explain the effect of parasitic inductance on switching transients. |  | CO4 | BTL 2 | Understanding |
| 4. | Why does parasitic inductance cause voltage overshoot during turn-off? |  | CO4 | BTL 2 | Understanding |
| 5. | What is parasitic capacitance? Give one common source in power circuits. |  | CO4 | BTL 1 | Remembering |
| 6. | Identify two effects of parasitic capacitance on high-speed switching. |  | CO4 | BTL 1 | Remembering |
| 7. | Explain how parasitic capacitance influences EMI generation. |  | CO4 | BTL 2 | Understanding |
| 8. | Describe the role of parasitic capacitance in common-mode noise. |  | CO4 | BTL 2 | Understanding |
| 9. | List two components commonly used in EMI filters. |  | CO4 | BTL 1 | Remembering |
| 10. | Define the purpose of an EMI filter in power electronics. |  | CO4 | BTL 1 | Remembering |
| 11. | Explain the difference between common-mode and differential-mode noise. |  | CO4 | BTL 2 | Understanding |
| 12. | How does a common-mode choke reduce EMI in high-frequency converters? |  | CO4 | BTL 2 | Understanding |
| 13. | Mention two PCB layout rules important for high-frequency circuits. |  | CO4 | BTL 1 | Remembering |
| 14. | What is the significance of minimizing loop area in high-frequency layout? |  | CO4 | BTL 1 | Remembering |
| 15. | Explain how trace impedance affects signal integrity in high-frequency PCBs. |  | CO4 | BTL 2 | Understanding |
| 16. | Describe the impact of ground bounce in high-frequency PCB designs. |  | CO4 | BTL 2 | Understanding |
| 17. | Define a conventional power loop in a power converter. |  | CO4 | BTL 1 | Remembering |
| 18. | List two limitations of conventional power loop design in high-frequency applications. |  | CO4 | BTL 1 | Remembering |
| 19. | Explain how stray inductance affects conventional power loop performance. |  | CO4 | BTL 2 | Understanding |
| 20. | Describe the typical current path in a conventional power loop. |  | CO4 | BTL 2 | Understanding |
| 21. | State one objective of high-frequency power loop optimization. |  | CO4 | BTL 1 | Remembering |
| 22. | Name two techniques used for minimizing parasitic inductance in power loops. |  | CO4 | BTL 1 | Remembering |
| 23. | Explain how loop compression improves switching performance. |  | CO4 | BTL 2 | Understanding |
| 24. | Describe the benefit of placing power switches and capacitors close together. |  | CO4 | BTL 2 | Understanding |
| PART- B |  |  |  |  |  |
| 1. | (i) Explain how parasitic inductance is formed in PCB traces and power loops. (ii) Discuss the impact of parasitic inductance on switching waveforms and EMI. | (8) (8) | CO4 | BTL 3 | Applying |
| 2. | (i) Describe how parasitic inductance causes voltage overshoot during MOSFET turn-off. (ii) Suggest practical methods to reduce parasitic inductance in PCB layout. | (9) (7) | CO4 | BTL 3 | Applying |
| 3. | (i) What are the primary sources of parasitic capacitance in a power converter circuit? (ii) Explain how parasitic capacitance contributes to common-mode noise and EMI. | (10) (6) | CO4 | BTL 3 | Applying |
| 4. | (i) Analyze the effect of parasitic capacitance on switching losses in wide bandgap devices. (ii) Discuss strategies to mitigate the impact of parasitic capacitance in PCB layout. | (8) (8) | CO4 | BTL 4 | Analyzing |
| 5. | (i) Explain the function of differential-mode and common-mode filters in EMI suppression. (ii) Design a basic EMI filter for a high-frequency buck converter with explanation. | (6) (10) | CO4 | BTL 4 | Analyzing |
| 6. | (i) Discuss the criteria for selecting inductors and capacitors for an EMI filter. (ii) Compare π-filter and LC filter topologies used in high-frequency converters. | (8) (8) | CO4 | BTL 4 | Analyzing |
| 7. | (i) Define common-mode and differential-mode noise with waveforms or diagrams. (ii) Suggest methods to reduce both types of noise in a switching power supply. | (10) (6) | CO4 | BTL 4 | Analyzing |
| 8. | (i) List and explain at least four PCB layout techniques to reduce EMI in high-frequency designs. (ii) Discuss the importance of minimizing loop inductance in gate drive and power loops. | (8) (8) | CO4 | BTL 3 | Applying |
| 9. | (i) Explain the significance of a continuous ground plane in high-speed PCB design. (ii) Describe the impact of ground bounce and how to minimize it. | (8) (8) | CO4 | BTL 3 | Applying |
| 10. | (i) Illustrate and explain the conventional power loop in a synchronous buck converter. (ii) Discuss the limitations of conventional loop designs in high-speed switching. | (8) (8) | CO4 | BTL 4 | Analyzing |
| 11. | (i) Describe how parasitic inductance and capacitance are distributed in a power loop. (ii) Explain the impact of these parasitic on high-frequency operation and EMI. | (7) (9) | CO4 | BTL 3 | Applying |
| 12. | (i) Explain the key objectives of optimizing power loops for wide bandgap power devices. (ii) Discuss the role of Kelvin source connection and its effect on gate voltage stability. | (8) (8) | CO4 | BTL 4 | Analyzing |
| 13. | (i) Describe the loop compression technique and its significance in high-frequency PCB layout. (ii) Explain the use of multi-layer PCB stack-up to minimize inductance. | (8) (8) | CO4 | BTL 4 | Analyzing |
| 14. | (i) Explain how decoupling capacitors help reduce high-frequency noise in power loops. (ii) Discuss placement strategies for bypass capacitors in high-speed PCB design. | (9) (7) | CO4 | BTL 4 | Analyzing |
| 15. | (i) Explain the importance of separating power and signal paths on a PCB. (ii) How does improper routing between power and signal traces affect performance and EMI? | (8) (8) | CO4 | BTL 3 | Applying |
| 16. | (i) Discuss the challenges in designing a PCB for mixed-signal systems (power + control). (ii) Suggest layout practices to prevent interference between analog and power sections. | (6) (10) | CO4 | BTL 3 | Applying |
| 17. | (i) Consider a 100 kHz buck converter using SiC devices—identify all major parasitic effects and how to mitigate them. (ii) Analyze an EMI filter and high-frequency PCB layout strategy suitable for this application. | (7) (9) | CO4 | BTL 3 | Applying |

| UNIT V APPLICATIONS OF WIDE BANDGAP DEVICES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Consumer electronics applications - Wireless power transfer applications - Electric vehicle applications - Renewable energy sources applications. |  |  |  |  |  |
| PART - A |  |  |  |  |  |
| Q.No | Questions |  | CO | BT Level | Competence |
| 1. | Define consumer electronics. |  | CO5 | BTL 1 | Remembering |
| 2. | Explain the role of SMPS in televisions or laptops. |  | CO5 | BTL 2 | Understanding |
| 3. | Give two examples of power converters used in consumer electronics. |  | CO5 | BTL 1 | Remembering |
| 4. | Why are compact and low-noise designs important in consumer electronics? |  | CO5 | BTL 2 | Understanding |
| 5. | What is the function of a power adapter in a mobile device? |  | CO5 | BTL 2 | Understanding |
| 6. | List any two features of energy-efficient power supplies in consumer devices. |  | CO5 | BTL 1 | Remembering |
| 7. | What is meant by wireless power transfer? |  | CO5 | BTL 1 | Remembering |
| 8. | State two types of wireless power transfer techniques. |  | CO5 | BTL 1 | Remembering |
| 9. | Explain the principle of inductive coupling in WPT. |  | CO5 | BTL 2 | Understanding |
| 10. | What are the main advantages of wireless charging in mobile devices? |  | CO5 | BTL 2 | Understanding |
| 11. | Mention two challenges in implementing high-power WPT systems. |  | CO5 | BTL 1 | Remembering |
| 12. | Describe how resonant coupling enhances wireless power efficiency. |  | CO5 | BTL 2 | Understanding |
| 13. | Name two types of electric vehicles. |  | CO5 | BTL 1 | Remembering |
| 14. | List two power electronic components used in EV drive trains. |  | CO5 | BTL 1 | Remembering |
| 15. | What is the function of an on-board charger in an electric vehicle? |  | CO5 | BTL 2 | Understanding |
| 16. | Explain how regenerative braking improves energy efficiency in EVs. |  | CO5 | BTL 2 | Understanding |
| 17. | Define battery management system (BMS) in EVs. |  | CO5 | BTL 1 | Remembering |
| 18. | Why are wide bandgap devices preferred in EV power converters? |  | CO5 | BTL 2 | Understanding |
| 19. | Name two renewable energy sources. |  | CO6 | BTL 1 | Remembering |
| 20. | What is the role of inverters in solar PV systems? |  | CO6 | BTL 2 | Understanding |
| 21. | State any two challenges in integrating renewable energy into the grid. |  | CO6 | BTL 1 | Remembering |
| 22. | Explain the function of a maximum power point tracker (MPPT) in solar systems. |  | CO6 | BTL 2 | Understanding |
| 23. | Mention two applications of wind energy conversion systems. |  | CO6 | BTL 1 | Remembering |
| 24. | Describe the need for energy storage in renewable energy applications. |  | CO6 | BTL 2 | Understanding |
| PART- B |  |  |  |  |  |
| 1. | (i) Explain the working principle of wireless charging in mobile phones, highlighting the role of Qi standards. (ii) Discuss the main challenges engineers face in optimizing coil design and placement for efficient wireless charging in consumer devices | (8) (8) | CO5 | BTL 4 | Analyzing |
| 2. | (i) Describe the benefits of wireless charging technology for consumer electronics in terms of user experience and device durability. (ii) Assess the impact of wireless charging adoption on the design and marketability of portable electronic devices | (9) (7) | CO5 | BTL 3 | Applying |
| 3. | (i) Discuss how wireless charger networking can improve the usability and efficiency of charging mobile devices in public spaces. (ii) Evaluate the factors influencing user-charger assignment in a wireless charger network, considering cost, location, and availability. | (8) (8) | CO5 | BTL 3 | Applying |
| 4. | Discuss the comparison the Basic Power Profile (BPP) and Extended Power Profile (EPP) in Qi wireless charging standards. | (16) | CO5 | BTL 4 | Analyzing |
| 5. | (i) Examine the potential health and safety concerns associated with the widespread use of wireless charging in household appliances. (ii)Discuss the market demand and cost-effectiveness of manufacturing wireless multi-device charging products for consumer use. | (8) (8) | CO5 | BTL 4 | Analyzing |
| 6. | (i) Explain the differences between inductive coupling, magnetic resonance coupling, and RF/microwave wireless power transfer techniques. (ii) Identify the key technical standards (such as Qi, PMA, A4WP) and their relevance to various wireless power transfer applications | (7) (9) | CO5 | BTL 4 | Analyzing |
| 7. | (i) Discuss the use of wireless power transfer in medical devices, such as implantable sensors, and its advantages over wired charging. (ii) Explain the challenges of implementing safety features in wireless charging systems for sensitive applications | (8) (8) | CO5 | BTL 3 | Applying |
| 8. | (i) Evaluate the application of wireless power transfer in Internet of Things (IoT) devices and sensor networks. (ii) Discuss the efficiency and power management challenges in deploying wireless power systems for distributed electronics. | (11) (5) | CO5 | BTL 3 | Applying |
| 9. | Assess the limitations of current wireless charging technologies in terms of range, alignment, and energy loss. | (16) | CO5 | BTL 4 | Analyzing |
| 10. | Describe the concept of simultaneous wireless information and power transfer (SWIPT) and its potential applications. | (16) | CO5 | BTL 4 | Analyzing |
| 11. | (i) Explain the principle of wireless charging for electric vehicles (EVs) using inductive power transfer. (ii) Discuss the benefits and challenges of implementing wireless charging infrastructure for EVs in urban settings. | (9) (7) | CO5 | BTL 3 | Applying |
| 12. | (i) Compare near-field and far-field wireless power transfer techniques for EV charging in terms of efficiency and practicality. (ii) Discuss the technological advancements required to enable dynamic (in-motion) wireless charging for EVs. | (8) (8) | CO5 | BTL 3 | Applying |
| 13. | (i) Evaluate the safety and regulatory considerations for deploying wireless charging systems for electric vehicles. (ii) Discuss how wireless charging can influence the adoption and usability of electric vehicles. | (9) (7) | CO5 | BTL 4 | Analyzing |
| 14. | (i) Assess the integration of wireless charging stations with renewable energy sources for sustainable EV charging. (ii) Examine the environmental impact of large-scale wireless charging infrastructure for electric vehicles. | (8) (8) | CO6 | BTL 3 | Applying |
| 15. | (i) Describe how wireless power transfer can enhance the flexibility and deployment of renewable energy systems in remote locations. (ii) Discuss the challenges in maintaining power quality and efficiency when integrating wireless power transfer with renewable sources. | (8) (8) | CO6 | BTL 3 | Applying |
| 16. | (i) Discuss the role of power electronics in enabling wireless power transfer for hybrid renewable energy systems. (ii) Evaluate the potential of wireless power transfer for off-grid renewable energy applications, such as remote sensors and microgrids. | (8) (8) | CO6 | BTL 3 | Applying |
| 17. | (i) Discuss the future prospects and barriers for large-scale adoption of wireless power transfer in renewable energy sectors. (ii) Examine the impact of wireless power transfer on the design and operation of distributed renewable energy networks. | (8) (8) | CO6 | BTL 4 | Analyzing |